Expt 1; To monitor temperature using IoT in Wokwi simulation.

#include "DHT.h"

#define DHTPIN 2

#define DHTTYPE DHT11

DHT dht(DHTPIN, DHTTYPE);

void setup() {

Serial.begin(9600);

dht.begin();

}

void loop() {

float temp = dht.readTemperature();

Serial.print("Temperature: ");

Serial.print(temp);

Serial.println(" °C");

delay(2000);

}

Expt2: To monitor temperature using ESP32 and upload (log) data to ThingSpeak cloud.

#include <WiFi.h>

#include "DHT.h"

#define DHTPIN 4

#define DHTTYPE DHT11

const char* ssid = "Wokwi-GUEST";

const char* password = "";

String apiKey = "YOUR_API_KEY";

const char* server = "api.thingspeak.com";

WiFiClient client;

DHT dht(DHTPIN, DHTTYPE);

void setup() {

Serial.begin(115200);

dht.begin();

WiFi.begin(ssid, password);

while (WiFi.status() != WL_CONNECTED) {

delay(500);

Serial.print(".");

}

Serial.println("WiFi Connected");

}

void loop() {

float temp = dht.readTemperature();

if (isnan(temp)) {

Serial.println("Sensor Error!");

return;

}

Serial.print("Temperature: ");

Serial.println(temp);

if (client.connect(server, 80)) {

String url = "/update?api_key=" + apiKey + "&field1=" + String(temp);

client.print(String("GET ") + url + " HTTP/1.1\r\n" +

"Host: " + server + "\r\n" +

"Connection: close\r\n\r\n");

}

client.stop();

delay(15000); // Important for ThingSpeak

}

Expt 3: To control two LEDs using ESP32 through Blynk web dashboard (laptop).

#define BLYNK_TEMPLATE_ID "YourTemplateID"

#define BLYNK_DEVICE_NAME "LED Control"

#define BLYNK_AUTH_TOKEN "YourAuthToken"

#include <WiFi.h>

#include <BlynkSimpleEsp32.h>

char ssid[] = "Wokwi-GUEST";

char pass[] = "";

#define LED1 2

#define LED2 4

void setup() {

Serial.begin(115200);

pinMode(LED1, OUTPUT);

pinMode(LED2, OUTPUT);

Blynk.begin(BLYNK_AUTH_TOKEN, ssid, pass);

}

// Button 1 → LED1

BLYNK_WRITE(V0) {

int value = param.asInt();

digitalWrite(LED1, value);

}

// Button 2 → LED2

BLYNK_WRITE(V1) {

int value = param.asInt();

digitalWrite(LED2, value);

}

void loop() {

Blynk.run();

}

Expt4: #define TRIG 9

#define ECHO 10

long duration;

float distance;

void setup() {

Serial.begin(9600);

pinMode(TRIG, OUTPUT);

pinMode(ECHO, INPUT);

}

void loop() {

digitalWrite(TRIG, LOW);

delayMicroseconds(2);

digitalWrite(TRIG, HIGH);

delayMicroseconds(10);

digitalWrite(TRIG, LOW);

duration = pulseIn(ECHO, HIGH);

distance = duration * 0.034 / 2;

Serial.print("Distance: ");

Serial.print(distance);

Serial.println(" cm");

delay(1000);

}

Expt 6: #define PIR 2

#define LED 13

void setup() {

pinMode(PIR, INPUT);

pinMode(LED, OUTPUT);

Serial.begin(9600);

}

void loop() {

int motion = digitalRead(PIR);

if (motion == HIGH) {

digitalWrite(LED, HIGH);

Serial.println("Motion Detected");

} else {

digitalWrite(LED, LOW);

Serial.println("No Motion");

}

delay(500);

}